# SP6001 Windows 11 Driver Setup guide

The ScanPrint SP6001 thermal printer uses the “JM Thermal Series Printer” driver.  Here are some basic installation steps for Windows 11 (Windows 10 should be very similar):

- After connect the power supply to the SP6001, connect the USB cable to your computer’s USB port

- In Windows Settings go to ‘Printers & Scanners’.  Click on ‘Add device’

- Click on ‘Add a new device manually’. This may take a few seconds to appear after step 2 above.

- Select ‘Add a local printer or network printer with manual settings’. Click Next

- In the “Choose a printer port” dialog;

  - Click on ‘Use an Existing port’, then click on the drop down list.

  - Select the ‘USB001’ port. (“ScanPrint SP6001” or “Virtual Printer Port for USB” will appear in brackets).

- In the “Install the printer Driver “dialog,

  - Select ‘Have Disk’     IMPORTANT

  - After clicking ‘Next’ navigate to the location of the supplied install disk contents, select the “Drivers\Windows 8” folder and then select the JMTPSB.INF file.

  - Click ’OK’ and then ‘Next’

- When the ‘Install the printer driver’ dialog re-appears select “JM Thermal Series Printer”

(note that you will be prompted to rename this (E.g. ScanPrint Receipt Printer) if you want)

- Click ‘Next’ and enter Administrator password if required, to enable Windows to install the driver.

- Applications should now be able to print using the SP6001 by selecting:  ‘JM Thermal Series Printer’ (or the name you changed to in step 7 above).